## 5.1 Thruster Specifications

Manufacturer data for the two thruster models under consideration is summarised below.

| Parameter | APISQUEEN U5 | APISQUEEN U15 |
| --- | --- | --- |
| Rated power | ≈800 W | ≈650 W |
| Voltage range | 12–24 V (up to 29 V charged) | 24–36 V |
| Thrust (single unit) | ≈5–7 kgf, voltage-dependent | Higher thrust/W; used at ≈9 kgf |
| Mass (single thruster, as used in range table) | ≈1.15 kg | ≈1.5 kg |
| Approx. unit cost | ≈₹8,000 | ≈₹13,500 |

Table 6. APISQUEEN U5 vs. U15 — key parameters used in the trade study (manufacturer listings; mass/cost figures cross-checked against whiteboard notes).

The design-session notes record that replacing a U5 pair with a U15 pair increases achievable runtime by a factor of ≈ 1.21x (17/14) for a comparable battery, i.e. the U15's higher propulsive efficiency (thrust delivered per watt at the speeds this vehicle operates at) more than offsets its lower rated power and higher unit cost and mass.

## 5.2 Battery and Propulsion Configuration Options

Six configurations were costed and weighed during the trade study, trading battery chemistry/capacity against thruster choice, achievable speed, endurance and cost.

| S.No. | Battery | Mass | Capacity | Thruster | Endurance | Speed | Est. cost |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1× LiPo | 2.6 kg | 22 Ah | 2× U5 (mini) | 39 min | 3.5 m/s | ₹17,400 |
| 2 | 2× LiPo | 5.2 kg | 44 Ah | 2× U5 (mini) | 78 min | 3.5 m/s | ₹34,800 |
| 3 | 2× LiPo | 4.8 kg | 32 Ah | 2× U5 (mini) | 56 min | 3.5 m/s | ≈₹32,840 |
| 4 | 2× Li-ion | 3.7 kg | 40 Ah | 2× U5 (mini) | 67 min | 3.5 m/s | ₹22,000 |
| 5 | 1× LiPo (10S) | 3.6 kg | 20 Ah | 2× U15 | 30 min | 6.5 m/s | ₹17,500 |
| 6 | 1× LiPo (10S) | 3.6 kg | 20 Ah | 1× U15 | 51 min | 4 m/s | ₹17,500 |

Table 7. Battery/propulsion configuration options from the initial trade study.

## 5.3 Range Sensitivity considering Two Batteries, Two Thrusters

A follow-on sensitivity study, holding the vehicle at two batteries and two thrusters, compared achievable one-way range and total system cost for the U5 (≈1.15 kg/thruster) and U15 (≈1.5 kg/thruster) options across four hull/loading variants. These figures are also transcribed from whiteboard notes and should be treated as indicative pending a written re-check.

| Variant | U5 range | U5 cost | U15 range | U15 cost |
| --- | --- | --- | --- | --- |
| 1 | 8.15 km | ≈₹51.5k | 9.89 km | ≈₹62.5k |
| 2 | 16.3 km | ≈₹51.5k | 19.79 km | ≈₹62.5k |
| 3 | 11.76 km | ≈₹49k | 14.28 km | ≈₹60k |
| 4 | 14 km | ≈₹60k | 17 km | ≈₹71k |

Table 8. Range/cost sensitivity, 2-battery / 2-thruster configurations

## 5.4 Selection Guidance

The U5 pairing is the lower-risk, lower-cost option and, at 3.5 m/s (Fn = 1.15), sits just past the resistance hump rather than deep in the planing regime as it is compatible with either a displacement or a semi-displacement hull. The U15 pairing costs and weighs more per thruster but reaches 5.5 m/s (Fn = 1.81) and delivers materially better runtime for a given battery once past the transition band provided the hull is shaped to generate the lift the Section 4 analysis shows is available at that speed. Because the range and cost advantage of U15 only fully materialises with a hull that planes, the thruster decision and the hull-form decision in Section 4 should be finalised together rather than independently: U5 + semi-displacement hull is the conservative pairing; U15 + planing/semi-planing hull is the higher-performance pairing carried as the current preferred direction.